LIGHTNING / THUNDER STORM

SCOPE

The purpose for this section is to ensure the overall Safety of Titan Drilling employees during a Lightning / Thunder Storm and applies to all Titan Drilling employees and other non – employees working on or around a Titan Drilling drill site.

RESPONSIBILITY

Site Supervisor - It is ultimately your responsibility to inform and train drill crews and all non-employees working on or around your drill site.

OPERATING DRILL DURING A LIGHTNING / THUNDER STORM

If operating a drill rig and lightning or thunder is closer than 15 kilometres, seek shelter immediately in a pick-up or similar type vehicle.

How to determine the distance of the lightning / thunder from your location

- Use the lightning detector which is made available at your area of operations;

- From the time the flash of light of lightning / thunder is spotted, count 1-one thousand, 2-one thousand, 3-one thousand and so on until the sound of the lightning / thunder is heard. Divide the number of seconds by 3 for kilometres.  Using this method will give a rough estimate as to the overall distance from your location to the actual location of the lightning / thunder strike.

Once in vehicle keep windows rolled up and refrain from touching any metal surfaces.

If there is no shelter crouch in the open, keeping twice as far away from trees as they are tall.  Place hands over ears to minimize hearing damage from lightning / thunder.  Avoid laying flat on the ground.

Never huddle in groups, make sure that each individual is several yards apart.

It is very important not to have anything metal on your person, as this will cause severe burns if struck by lightning / thunder

INJURED PERSONS

Injured persons do not carry an electrical charge and can be handled safely.

Apply first aid procedures immediately if you are qualified to do so.

Know your emergency telephone numbers.

DETERMINING WHEN IT IS SAFE TO CONTINUE WORKING

Once seeking shelter from the lightening / thunder storm wait for 30 minutes from the last known lightning / thunder flash to continue working.